|  | Morden Networking Practical # 1 |  |  |
| --- | --- | --- | --- |
| Name | Ninad Karlekar | Roll Number | 22306A1012 |
| Subject/Course: | Morden Networking | Class | M.Sc. IT – Sem II |
| Topic | Using the AS_PATH attribute of BGP | Batch | 1 |
| Topic: Using the AS_PATH attribute (Autonomous System path attribute) of BGP(Border Gateway Protocol) |  |  |  |
| - Open GNS3 and create a new project. - Drag and drop 3 routers as R1, R2, R3 - Right click on router -> configure -> select router name -> slots -> Select slot 1 as NM-4T - Connect routers: R1 and R2 ==> s1/0 ¦¦ R1 and R3 ==> s1/1 - Label topology with given labels below and click on run |  |  |  |
| CONSOLE (On Router console type following commands one by one.) |  |  |  |
| R1 console R1#conf t R1(config)#int s1/0 R1(config-if)#ip add 192.168.1.1 255.255.255.0 R1(config-if)#no sh R1(config-if)#int s1/1 R1(config-if)#ip add 172.24.1.1 255.255.255.0 R1(config-if)#no sh |  |  |  |
| R2 console R2#conf t R2(config)#int s1/0 R2(config-if)#ip add 192.168.1.2 255.255.255.0 R2(config-if)#no sh |  |  |  |
| R3 CONSOLE R3#conf t R3(config)#int s1/1 R3(config-if)#ip add 172.24.1.3 255.255.255.0 R3(config-if)#no sh |  |  |  |
| loopback address |  |  |  |
| R1 CONSOLE int lo0 ip add 10.1.1.1 255.255.255.0 |  |  |  |
| R2 CONSOLE int lo0 ip add 10.2.2.2 255.255.255.0 |  |  |  |
| R3 CONSOLE int lo0 ip add 10.3.3.3 255.255.255.0 |  |  |  |
| bgp protocol |  |  |  |
| R1 CONSOLE router bgp 100 neighbor 192.168.1.2 remote-as 200 neighbor 172.24.1.3 remote-as 300 network 10.1.1.0 mask 255.255.255.0 |  |  |  |
| R2 CONSOLE router bgp 200 neighbor 192.168.1.1 remote-as 100 network 10.2.2.0 mask 255.255.255.0 |  |  |  |
| R3 CONSOLE router bgp 300 neighbor 172.24.1.1 remote-as 100 network 10.3.3.0 mask 255.255.255.0 |  |  |  |
| do sh ip route |  |  |  |
| OUTPUT |  |  |  |
| From R2(COMPANY) to R3(CUSTOMER) do ping 10.3.3.3 source lo0 |  |  |  |
| From R3(CUSTOMER) to R2(COMPANY) do ping 10.2.2.2 source lo0 |  |  |  |